Information required on as constructed drawings for taking in charge of developments by South Dublin County Council

In conjunction with requirements for taking in charge, please submit as constructed drawings relating to the development on disc to Autocad 2004 or below, showing the following information:

The Council requires that, to the greatest extent possible, there is clear, discernible and practical delineation between areas that will be taken in charge(in yellow) and those that will remain the responsibility of householders or combinations of householders who may assemble in their common interest, for example through a management company.  Areas to remain in the control of such management companies should be clearly hatched.

- A clear layout using Ordnance Survey Ireland background detail, showing the roads network, road names, house locations and postal addresses and indicating the estate boundary.

- Show drainage services layout, in particular, manhole locations and pipeline locations for foul sewer (coloured red), surface water sewer (coloured green) and their connection points to the existing public sewers.

- Show manhole reference numbers, cover levels and invert levels to Malin Head datum, pipe sizes and gradients, tabulated off the area of the layout on the drawing for clarity. Manhole reference numbers only, to be also displayed on the layout. National Grid Co-ordinates of each manhole cover to be provided.

- Watermain layout (coloured blue) showing locations of hydrants, sluice valves, and air valves, also pipe sizes and their connection points to the existing public service.

- Only services which are being proposed for Taking in Charge should be shown using the prescribed colours, with existing services acting as outfalls and connection points, shown black.

A typical example drawing, in jpg format to illustrate the standard and the layout type which is required, is also available to download, and should be referred to in conjunction with this document.